MSDOS 2.0 Installation kit

The MS-DOS Installation kit is available on 5 1/4 inch MS-

DOS disks or 8 inch MS-DOS disks.  It consists of the MS-DOS

release software in addition to an evaluation copy of the

Microsoft Macro Assembler.  In addition to software and

documentation on disk, the installation kit includes three

manuals.

1. MS-DOS 2.0 User's Guide, a user's introduction to DOS

commands and utilities.

2. MS-DOS 2.0 Programmer's Reference,  a summary of all

PUBLISHED system calls and  DOS structures.

3. MSDOS Macro Assembler Manual Documentation for the Macro

Assembler, Cross reference utility, Linker, LIBrarian and

DOS debugger. Included with the Macro Assembler Evaluation

package.

Common Questions and Answers Regarding Installation of MS-

DOS.

Q. What Kind of Development Machine do you recommend?

A. Microsoft recommends that an MS-DOS machine be used as a

development tool in preparing software for your target

machine.  Use of an MS-DOS machine will avoid problems with

incompatible assemblers and object module formats.  You will

probably want to be able to read standard MS-DOS disk

formats.  Use of the development machine to build a system

disk for the target machine simplifies the implementation

process.

Q. Are Sources Available?

A. Sources for most* of MS-DOS are available to licensees of

the product for an additional source fee.  These sources are

used by Microsoft on a mainframe computer for assembling MS-

DOS using an internally developed Cross Assembler.

Guarantees as to their assembleability on other systems can

not always be made.

* Sources for the linker and some other utilites are not

included in the MS-DOS source package.

Q. What Language is MS-DOS Written In?

The source code for MSDOS and most utilities furnished is

written in 8086 Assembler code compatible with the macro

facility found in the  Microsoft Macro Assembler for the MS-

DOS Operating System.  The format of the object modules

provided is a subset of INTEL's and may not be compatible

with non-Microsoft linkers.

Q. What will an MS-DOS Implementor need to know:

A. To Install MS-DOS you will need to be  familiar with

systems programming in 8086 assembler language. This means

you have experience writing device drivers for disk drives,

keyboards, printers, etc.

Q. How Long Does an MS-DOS Implementation Take:

A. This depends on whether the low level I/O routines are

already written.  The record time for getting MS-DOS up and

running on a new machine is one weekend.  In this case, all

of the low level routines were in ROM.  Typically to do a

complete MS-DOS implementation that one would ship to a

customer takes two or more man months.

Q. Are there any consultants who do MS-DOS implementations?

A. Microsoft can supply a list of consultants who have done

MS-DOS implementations in the past.  Microsoft can not make

any recommendations regarding the competence of these

consultants.

MS-DOS Overview

An MS-DOS implementation consists of the following

components:

1. The default device drivers often called  BIOS (from Basic

Input Output System) a  collection of hardware specific

device drivers which must be written by the OEM.  The

default device drivers are called by MS-DOS to handle I/O

requests.  These device drivers may be partly ROM resident

or may be completely RAM resident.

2. SYSINIT, a Microsoft supplied module which is linked to

the BIOS and takes care of system initialization.  The file

consisting of the combined BIOS and SYSINIT is named IO.SYS.

3. MSDOS.SYS, the Microsoft supplied hardware independent

Disk Operating System.

4. COMMAND.COM, the command interpreter program which reads

and interprets user keyboard input and executes "built-in"

commands like DIR or initiates external programs.

5. Bootstrap Loader, the OEM supplied module which loads the

IO.SYS and MSDOS.SYS files into memory (MSDOS.SYS may

optionally be loaded by IO.SYS).

6. A system ROM which will may optionally perform

diagnostics on power-up or reset and will load the boot

sector into RAM and transfer control to it.

7. MS-DOS utilities including FORMAT for which a Microsoft

module is provided and a hardware-specific section must be

written by the OEM.

MSDOS MEMORY MAPS:

During the system initialization process, there are a series

of memory arrangements as the bootstrap loader gets loaded

into memory, as the IO.SYS and MSDOS.SYS files are loaded and

finally as the SYSINIT module which is part of IO.SYS installs

MSDOS, installable drivers and then COMMAND.COM.  The memory

maps are shown below each representing a moment in the

initialization process.

Memory Organization in an MS-DOS system after the system

ROM loads the Boot Sector.

+-----------------------------+

|                             |    High Memory

|                             |

|                             |

|                             |

+-----------------------------+

|                             |

|  bootstrap loader           |    loaded at arbitrary

|                             |    location by system ROM

+-----------------------------+ -  XX:00

|                             |

|                             |

|                             |

|                             |

|                             |

|                             |

|                             |

|                             |

|                             |

|                             |

+-----------------------------+  Segment 40H

| interrupt vector table      |

+-----------------------------+  -0:00

Memory Map After Bootstrap Loader

Loads IO.SYS and MSDOS.SYS

+-----------------------------+

|                             |    High Memory

|                             |

|                             |

|                             |

+- - - - - - - - - - - - - - -+

|                             |

|   spent  bootstrap loader   |    loaded at arbitrary location

|                             |    by system ROM

+- - - - - - - - - - - - - - -+

|                             |

|                             |

|                             |

|                             |

+-----------------------------+

|                             |

|         MSDOS.SYS           |    MSDOS disk operating system

|                             |    in temporary location

|                             |

+-----------------------------+

|         SYSINIT             |    MS supplied system initiali-

+- - - - - - - - - - - - - - -+    zation module (Part of IO.SYS)

|                             |

|           IO.SYS            |    IO.SYS may begin at any location

|                             |    above IVT

+-----------------------------+

| interrupt vector table      |

+-----------------------------+

Memory Map During System Initialization

performed by SYSINIT module

+-----------------------------+

|          SYSINIT            |    High Memory

| relocates itself to highmem |

+-----------------------------+

|                             |

|                             |

|                             |

|                             |

+-----------------------------+

|                             |

| dos structures, buffers and |    SYSINIT installs non-BIOS

|    installable drivers      |    drivers and buffers here

+-----------------------------+

|                             |

|         MSDOS.SYS           |    SYSINIT moves MS-DOS

|     relocated by SYSINIT    |    down to OEM designated

|    to fill hole in memory   |    final DOS locationF

|                             |

+-----------------------------+

|                             |

|         IO.SYS              |

|                             |

+-----------------------------+

| interrupt vector table      |

+-----------------------------+

Memory Map After SYSINIT Installs Command Interpreter

This represents the normal configuration

During MS-DOS Operation

+-----------------------------+

|          COMMAND.COM        |    High Memory

|           transient         |

+-----------------------------+

|                             |    Start of TPA will vary

|    Transient Program Area   |    with # of drivers, buffers

|         (TPA)               |    etc.

+-----------------------------+

|    COMMAND.COM Resident     |    COMMAND.COM loads its

+-----------------------------+    transient into top of

|                             |    largest available memory

| dos structures, buffers and |

|    installable drivers      |

+-----------------------------+

|                             |

|         MSDOS.SYS           |

|     relocated by SYSINIT    |

|    to fill hole in memory   |

|                             |

+-----------------------------+--- OEM specified Final

|                             |    DOS Location

|         IO.SYS              |

|   default device drivers    |

+-----------------------------+

| interrupt vector table      |

+-----------------------------+

MS-DOS Development Tools

An Assembler, text editor, linker and other utilities are

required for completing an MS-DOS implementation.  Some of

these utilities are provided with the MS-DOS 2.x release,

but it is assumed that your development machine will have

the standard utilities included.

The EDLIN Text Editor

The EDLIN Text Editor provided with the MS-DOS release for

your development machine may be used to prepare assembler

source files.  The version of EDLIN on the MS-DOS 2.x

release disks is for version 2.x only and will not run under

MS-DOS 1.x.

The Microsoft Macro Assembler

An Evaluation Copy of the Microsoft Macro Assembler is

provided with the MS-DOS Installation Kit.  The assembler is

not actually a part of MS-DOS and must be licensed

separately if you wish to ship it to your customers.  CREF,

included with the Macro Assembler can be used to make cross

reference listings of assembler programs.  The Macro

Assembler package will run under all MS-DOS  1.x / 2.x

systems.  The output of the assembler is a relocatable

object module (.OBJ file type) which conforms to a subset of

the INTEL MCS 86 Object Module Format description.

The Microsoft Linker:

Your MS-DOS development machine should include a linker at

the proper MS-DOS release level.  The Linker on the MS-DOS

2.x release diskettes will not run under MS-DOS 1.x. Use

LINK.EXE  for linking OBJECT modules (.OBJ files) produced

by the Macro Assembler.  The output of the linker is a file

of .EXE format.  This is a non-executable file and requires

a special loader which is part of MS-DOS.  Refer to the MS-

DOS Programmer's Reference Chapter 5 for details on EXE

formats.

DEBUG

DEBUG.COM is the MS-DOS debugger.  The version supplied with

the MS-DOS 2.x release will not run under MS-DOS 1.x.  The

debugger has the capability to do absolute disk reads and

writes.  You may use this facility to write your bootstrap

loader program into the boot sector.EDLIN.COM A line

oriented text editor for writing assembler routines.

EXE2BIN

The EXE2BIN.COM  utility supplied with your MS-DOS

development machine must be used for converting the output

of the linker (.EXE files) to a pure binary (core image)

format.  As above, the EXE2BIN version on the release disks

will run under MS-DOS 2.x only.

FORMAT

The FORMAT program supplied with your MS-DOS development

machine may be used to format a boot disk for the target

machine.

MS-DOS Implementation Overview

A Step By Step Procedure for DOS implementation.  If you are

using an MS-DOS machine for development, and your target

machine will read MS-DOS standard disk formats, you may

follow this general outline for implementing MS-DOS.

1. Using a text editor on your development machine, write

the 8086 assembler  source code for the default device

drivers.  The default device drivers (sometimes referred to

as the BIOS) are the hardware specific routines which will

be accessed by MS-DOS to perform I/O.  There MUST a minimum

of 5 device drivers, four "character" type drivers  and one

block device driver (the disk drive on most systems). In

addition to the device drivers, you may want to include some

code  for hardware initialization.  The source file should

be named IO.ASM and must not contain a STACK segment.

SEE:

Writing A Device Driver

Sample Device Drivers

SKELIO.ASM     disk file

2. Use MASM.EXE, the macro assembler which is provided with

the MS-DOS Installation Kit to assemble your device driver

code.    MASM will create a file called IO.OBJ which is in a

special Object Module Format (a subset of Intel's Object

Module Format).

SEE:

The Microsoft Macro Assembler Manual

3. Use LINK.EXE the Microsoft Linker to link your IO.OBJ

module to SYSINIT.OBJ and SYSIMES.OBJ.  The modules should

be linked in that order. The output file should be named

IO.EXE.  You will get a message "Warning, no STACK segment"

while linking the module. This is a warning message and is

normal. Ignore it.

SEE:

MS-DOS User's Guide (Link Section)

4. The file created in step 4 is a .EXE file which is in a

special format recognized by the EXE Loader.  Since the EXE

loader is not available at BOOT time, the IO.EXE file must

be converted to a pure binary core image file.  This is done

with the EXE2BIN utility provided with the MS-DOS release on

your development machine.  The EXE2BIN utility will have to

resolve far references in the IO.EXE code.  In order to

resolve these references, EXE2BIN must know exactly where

the code will reside in memory.  It will prompt you for a

"BASE ADDRESS" which is the absolute segment address where

the code is to begin.  The resultant file must then be named

IO.SYS.  EXE2BIN will change the name if the command line is

typed:

EXE2BIN   IO.EXE    IO.SYS

Note: It is the responsibility of your bootstrap loader to

locate IO.SYS at the location you specified as the base

address.  The location itself is entirely arbitrary since

one of the parameters that your default device driver code

passes to SYSINIT is the location of the first device

driver.  The only special addresses as far as MS-DOS is

concerned are interrupts 20H through 3FH in the interrupt

vector table.

SEE:

MS-DOS User's Guide (EXE2BIN)

5.   Write a bootstrap loader program using the text editor.

Name this  file BOOT.ASM (It is assumed that you have some

kind of system ROM which will load the first sector of the

system disk into memory upon power on or reset). The

bootstrap loader would ideally fit into the first sector of

the boot diskette. Microsoft recommends that in addition to

the loader, the first sector of the disk contain a table

describing the disk format as well as some OEM information.

The bootstrap loader's responsibility is to load IO.SYS and

MSDOS.SYS into memory.  MSDOS can be located any place in

memory above IO.SYS. (IO.SYS can alternately be used to load

MSDOS.SYS). The task of loading IO.SYS and MSDOS.SYS is made

somewhat simpler by the requirement that these two files are

always the first files on the diskette. This means that a

bootstrap loader could just load a series of consecutive

sectors into memory.

SEE:

Writing a Device Driver (details of

BPB table in boot sector)

6. Assemble, Link, and EXE2BIN the BOOT.ASM file producing

BOOT.BIN.

7. Using your MSDOS development machine, format a non system

diskette.

8. Using the standard MSDOS COPY command, copy IO.SYS,

MSDOS.SYS, and the 2.0 version of COMMAND.COM to the just

formatted diskette.

9. Using DEBUG.COM on the development machine, load BOOT.BIN

and write it to the first sector of the formatted diskette.

This can be done as follows:

A:DEBUG

-N B:BOOT.BIN -L (loads B:BOOT.BIN starting at 100 hex)

-W 100 0 0 1  (writes data to drive 0 sector 0

writing 1 sector)

-Q            (Quits Debug)

SEE:

Microsoft Macro Assembler Manual

Debug Section

10. The formatted diskette should now be bootable by your

system.

11. Write the source code for the hardware specific part of

the FORMAT program.  Assemble and link this to the Microsoft

supplied FORMAT.OBJ and FORMES.OBJ.  Use EXE2BIN to convert

the FORMAT.EXE file produced to FORMAT.COM.

SEE:

Writing Format

GENFOR.ASM     Disk file of sample OEM  format

module

MS-DOS User's Guide (Format Command)

MS-DOS Programmer's Reference (disk formats)

Writing The Default Device Drivers

One of the most powerful features present in MS-DOS is the

ability to include new devices such as printers, plotters,

or mouse input devices without rewriting the BIOS.  The MS-

DOS BIOS is said to be "configurable" since new drivers can

be freely added and existing drivers can be preempted.

There are two types of drivers which differ very slightly,

the Default Drivers which make up the IO.SYS file and the

Installable Drivers which are added at boot time via the

"DEVICE =" entry in the special file, CONFIG.SYS.

AT BOOT Time, a minimum of five devices must be present.

These devices are in a linked list, the "header" of each one

containing a DWORD pointer to the next.  The last device in

the chain has and end-of-list marker of -1, -1.

Each driver  in  the  chain  has two entry points; the

STRATEGY entry point and the and  the  INTERRUPT entry

point.  The  2.0  DOS does not  really make use of two entry

points (it simply calls strategy, then immediately calls

interrupt).

The dual  entry point scheme  is  designed  to facilitate

future multi-tasking versions of MS-DOS.  In multi-tasking

environments  I/O  must be asynchronous,  to  accomplish

this  the  strategy  routine will be called to queue

(internally)  a  request  and  return quickly.  It  is  then

the  responsibility  of  the interrupt routine to perform

the actual I/O at interrupt time by picking requests  off

the  internal  queue  (set  up  by the strategy routine),

and process  them. When  a  request  is  complete, it is

flagged  as  "done"  by the interrupt routine.  The DOS

periodically scans the  list  of  requests  looking  for

ones flagged as  done,  and  "wakes up" the process waiting

for the completion of the request.

In order for requests to be  queued  as  above  it  is  no

longer sufficient  to pass I/O information in registers,

since many requests may be  pending  at  any  one  time.

Therefore the new  device  interface uses data "packets" to

pass request information.  A device is called with a pointer

to  a  packet, this packet  is  linked  into  a  global

chain of all pending I/O requests maintained by the DOS.

The  device  then  links the packet  into  its  own  local

chain of requests for this particular  device. The  device

interrupt  routine  picks requests of  the  local  chain

for  processing. The  DOS  scans  the global chain looking

for completed requests.  These  packets  are  composed  of

two pieces, a static  piece  which  has the same  format for

all requests (called the  static request header), which  is

followed by  information  specific  to the  request. Thus

packets  have  a  variable size and format.

At this point it should be  emphasized  that  MS-DOS  2.0

does not  implement most of these features as future

versions will.  There is no global or local queue. Only  one

request is pending at any one time, and the DOS waits for

this current request to be completed.  For 2.0

implementations, the strategy  strategy routine  must

simply  store  the address of the packet at a fixed

location, and the  interrupt  routine, which is called

immediately after strategy should process this  packet  by

completing  the  request  and returning.  Remember:  the DOS

just calls the strategy  routine  and  then immediately

calls  the  interrupt  routine. It is assumed that the

request is completed when the interrupt  routine  returns.

FORMAT OF A DEVICE DRIVER

A device  driver  is  simply  a  relocatable  memory image

with all of the code in  it  to  implement  the  device

(like a .COM  file,  but  not ORGed at 100 Hex).  In

addition it has a special header at the front of it  which

identifies  it  as a device,  defines  the  strategy  and

interrupt entry points, and defines various  attributes. It

should  also  be  noted that there are two basic types of

devices.

The first  are  character devices.  These are devices

which are designed to do character  I/O  in  a  serial

manner  like CON, AUX,  and  PRN. These devices are named

(ie.  CON, AUX, CLOCK, etc.), and users may open channels

(FCBs)  to  do  I/O to them.

The second  class  of  devices  is  block  devices.  These

devices are the "disk drives"  on  the  system,  they  can

do random I/O  in  pieces  called  blocks  (usually  the

physical sector size) and  hence  the  name. These  devices

are  not "named" as  the  character  devices  are, and

therefore cannot be "opened" directly. Instead  they  are

"mapped"  via  the drive letters (A,B,C, etc.).

Block devices  have  units.  In other words a single driver

may be responsible for one or more  disk  drives. For

instance block device driver ALPHA (please note that we

cannot actually  refer  to  block  devices  by  a  name!)

may be responsible for  drives A,B,C  and  D, this simply

means that it has four units (0-3) defined and therefore

takes  up  four drive letters. Which units correspond to

which drive letters is determined by the position  of  the

driver  in  the  chain of all  drivers: if  driver  ALPHA is

the first block driver in the device chain, and it defines 4

units (0-3),  then  they will be  A,B,C  and  D. If  BETA

is the second block driver and defines three units (0-2),

then  they  will  be  E,F  and G and  so  on. MS-DOS  2.0 is

not limited to 16 block device units, as  previous  versions

were. The  theoretical  limit is 63  (2^6  -  1),  but  it

should be noted that after 26 the drive letters get a little

strange (like ] \  and  ^). NOTE: Character devices  cannot

define multiple units (this because they have only one

name).

DEVICE DRIVER HEADER FORMAT

+-------------------------------------+

| DWORD Pointer to next Device        |

| (Set to -1,-1 on installable        |

|  devices)                           |

+-------------------------------------+

| WORD Attributes                     |

|  Bit 15 = 1 if char device or 0     |

|           if block device           |

|  if bit 15 is 1:                    |

|     Bit 0 = 1 if Current sti device |

|     Bit 1 = 1 if Current sto device |

|     Bit 2 = 1 if Current NUL device |

|     Bit 3 = 1 if Current CLOCK dev  |

|     Bit 4 = 1 if Special device     |

|  bit 14 is the IOCTL bit            |

|  bit 13 is NON IBM Format Bit       |

+-------------------------------------+

|  WORD Pointer to Device STRATEGY    |

|       Entry Point                   |

+-------------------------------------+

|  WORD Pointer to Device INTERRUPT   |

|       Entry Point                   |

+-------------------------------------+

|  8 BYTE Character Device Name Field |

|    Left Justified Space Filled      |

|    Upper Case Characters            |

|                                     |

|  For Block Devices, first byte is   |

|  Number of Units Defined            |

+-------------------------------------+

Note that the device entry points are words.  They must be

offsets from the same segment number used to point to this

table.  Ie. if XXX:YYY points to the start of this table,

then XXX:STRATEGY and XXX:INTERRUPT are the entry points.

A word about the Attribute  field. This field  is  used most

importantly  to  tell  the  system whether this device is a

block or character device (bit  15). Most  of  other  bits

are used  to  give  selected character devices certain

special treatment (NOTE: except for bit 13, these bits mean

nothing on a  block  device).  Let's say  a  user  has  a

new device driver which he wants to be the standard input

and  output. Besides  just installing the driver  he  needs

to  tell  SYSINIT (and the DOS) that he wishes his new

driver to override  the  current sti (standard input) and

sto (standard output) - the CON device.

This  is  accomplished  by  setting the attributes to the

desired characteristics,  so she  would  set Bits 0  and 1

to 1 (note that they are separate!!).  Similarly a  new

CLOCK  device  could  be  installed  by  setting  that

attribute, see  the  section   on the CLOCK device. NOTE:

that although there  is  a  NUL  device attribute,  the NUL

device  cannot  be  re-assigned. This  attribute  exists for

the DOS so that it can tell if the  NUL  device  is  being

used.

The NON  IBM  FORMAT  bit  applies  only  to block devices

and effects the operation of the  get  BPB  device  call

(see below).

The other  bit  of  interest  is  the  IOCTL bit which has

meaning on character or block devices.   This  bit  tells

the DOS whether  this  device  can handle control strings

(via the IOCTL system call, function 44H).   The IOCTL

functions allow data to  be  sent  and  received by the

device  itself for its own use (to set baud rate, stop bits,

form length etc., etc.), instead of  passing  data  over the

device  channel  as  a  normal  read  or write does.  The

interpretation of the passed information is up to the

device, but it MUST NOT simply be treated as a normal I/O.

If a driver cannot  process  control  strings,  it  should

initially set  this  bit  to  0.  This tells the DOS to

return an error if an attempt is made  (via  IOCTL  system

call)  to send or  receive  control  strings  to  this

device.  A device which can process control  strings  should

initialize  it  to 1.  For  drivers  of  this  type,  the

DOS can make calls to the IOCTL INPUT  and  OUTPUT  device

functions  to  send  and receive IOCTL strings (see  IOCTL

in  the  SYSTEM-CALLS document).

The SPECIAL  bit  applies  only  to  character drivers and

more particularly to  CON  drivers.  The  new  2.0

interface is a  much  more  general  and  consistent

interface than the old 1.25 DOS interface.  It allows for a

number of the new  features of  2.0.  IT  IS  ALSO SLOWER

THAN 1.25 IF OLD STYLE "SINGLE BYTE" SYSTEM CALLS ARE MADE.

To make  most  efficient use of  the  interface  all

applications  should  block their I/O as much as possible.

This  means  make  one  XENIX  style system call  to  output

X  bytes  rather  than X system calls to output one byte

each.

Putting a  device  channel  in RAW mode  (see  the IOCTL

DOS  function call, 44H)  provides  a  means  of  putting

out characters even FASTER  than  1.25. To  help  alleviate

the CON output  speed  problem  for  older  programs which

use the 1 - 12 system calls  to  output  large  amounts  of

data  the SPECIAL bit  has  been implemented.  If this bit

is 1 it means the device is the  CON  output  device,  and

has  implemented an interrupt  29  Hex  handler,  where  the

29 Hex handler is defined as follows:

CONSOLE OUTPUT via INT 29H handler

Input:       Character in AL

Function:    output the character in al to the user

screen.

Output:      None

Registers:  all registers except bx must be preserved.

No registers except for AL have a known or

consistent value.

If a  character  device  implements  the  SPECIAL  bit, it

is the responsibility of the  driver  to  install  an

address at the  correct  location in the interrupt table for

interrupt 29 Hex as part of  its  INIT  code. IMPLICATION:

There  can be only  one  device  driver  with  the SPECIAL

bit set in the system.  There is no check to insure this

state.

WARNING: THIS FEATURE WILL NOT BE SUPPORTED IN FUTURE

VERSIONS OF THE  OPERATING  SYSTEM. IMPLICATION:  Any

application (not device driver)  which  uses  INT  29H

directly  WILL NOT WORK ON FUTURE VERSIONS.

In  order  to  "make"  a  device  driver  that SYSINIT can

install, a .BIN (core image) or .EXE  (non-IBM  only)*

format  file must be  created  with  the  above  header  at

the start.  The link field should be  initialized  to  -1

(SYSINIT  fills  it in).  Device Drivers which are part of

the BIOS should be have their headers point to the next

device in the list with the last header initilized to -1,-1.

The BIOS must be a .BIN (core image) format file.

* The EXE file loader located in COMMAND.COM in the

IBM version.

The  attribute  field  and  entry  points  must  be set

correctly, and if the device is a character device,  the

name field must  be  filled  in  with  the  name (if a block

device SYSINIT will fill in  the  correct  unit  count).

This  name can be  any  8  character  "legal" file name.  In

fact SYSINIT always installs character devices at the start

of  the  device list, so  if  you  want  to  install  a new

CON device all you have to do is name it "CON".  The new

one  is  ahead  of  the old one  in  the  list  and  thus

preempts the old one as the search for devices stops on  the

first  match.  Be  sure  to set the sti and sto bits on a

new CON device!

NOTE:  Since  SYSINIT  may  install  the  driver anywhere,

you must be very careful about  FAR  memory  references. You

should NOT  expect  that  your  driver will go in the same

place every time (The default BIOS  drivers  are  exempted

from this of course).

INSTALLATION OF DEVICE DRIVERS

Unlike past versions, MS-DOS 2.0 allows new device  drivers

to be installed dynamically at  boot  time.  This  is

accomplished by the SYSINIT module supplied by  Microsoft

which reads  and  processes  the CONFIG.SYS file.  The

default driver source file is assembled and linked to

SYSINIT.OBJ and SYSIMES.OBJ.  The resultant .EXE file must

be converted via EXE2BIN to create the file IO.SYS. which

should be the first file appearing on the system diskette.

When block drivers are  installed  as  above    the  logical

drive letters  are  assigned  in  list  order, thus the

driver which is to have logical A must  be  the  first  unit

of  the first  block  device  in  the  list. The  order of

character devices is also important.  There must be at least

4 character devices defined  at  boot time which must be the

first four devices (of either  type),  the  first  will

become  standard  input, standard output,  and  standard

error output.  The second will become standard auxiliary

input and  output,  the  third  will become standard  list

output,  and  the forth will become the date/time (CLOCK)

device. The linked list of device drivers must must look

like this:

->CON->AUX->PRN->CLOCK->any other block or character

devices followed by BLOCK devices.

THE CLOCK DEVICE

One of  the  most  popular add on boards seems to be the

"Real Time CLOCK Board".  To allow these boards  to  be

integrated into the  system  for TIME and DATE, there is a

special device (determined by the attribute word) which is

the CLOCK  device.  In all  respects  this  device  defines

and performs functions like any other character device (most

functions will  be  "set done bit,  reset  error  bit,

return). When a read or write to this device occurs, exactly

6 bytes are transferred.   Reading  the CLOCK device  gets

the  date and time, writing to it sets the date and time.

BINARY DATE TIME FORMAT

byte 0   byte 1    byte 2    byte 3  byte 4   byte 5

+--------+--------+---------+--------+--------+---------+

|        |        |         |        |        |         |

|days since 1-1-80| minutes |  hours | sec/100| seconds |

|        |        |         |        |        |         |

+--------+--------+---------+--------+--------+---------+

THE DEVICE DRIVER STRATEGY ROUTINE

As mentioned before, the first entry point is the strategy

routine.  Currently, the strategy routine has one purpose,

to pass to the device driver a pointer to "request packet"

built by MS-DOS.  ON entry, ES:BX point to the packet and

should be stored by the driver.  The Strategy Routine should

then do a FAR RETURN.  MS-DOS will immediately call the

INTERRUPT entry point.  The "interrupt" routine  is called

with no parameters, its primary function is to perform  the

operation  based  on  the  data in the request packet in

memory and then return to the DOS.

THE DEVICE DRIVER INTERRUPT ROUTINE

A device driver must perform the following functions:

Command Function  Code

0  INIT

1  MEDIA CHECK (Block only, NOP for character)

2  BUILD BPB        "  "   "  " "

3  IOCTL INPUT (Only called if device has IOCTL)

4  INPUT  (read)

5  NON-DESTRUCTIVE INPUT NO WAIT (Char devs only)

6  INPUT STATUS                    "   "  "

7  INPUT FLUSH                     "   "  "

8  OUTPUT (write)

9  OUTPUT (Write) with verify

10  OUTPUT STATUS                   "   "  "

11  OUTPUT FLUSH                    "   "  "

12  IOCTL OUTPUT (Only called if device has IOCTL)

REQUEST PACKET  FORMAT

The Interrupt routine actually performs the request

described by the packet and returns to MS-DOS upon

completion of the request.  The Request Packet has a format

which must be recognized by the device driver.  It consists

of a fixed length header followed by various parameters

depending upon which of the 13 calls are being made.

NOTE: It is the driver's responsibility to preserve machine

state.

REQUEST PACKET HEADER ->

+-----------------------------+

| BYTE length of record       |

|  Length in bytes of this    |

|  Request Structure          |

+-----------------------------+

| BYTE unit code              |

|  The subunit the operation  |

|  is for (drive)             |

|  no meaning on character    |

|  devices                    |

+-----------------------------+

| BYTE command code           |

+-----------------------------+

| WORD Status                 |

+-----------------------------+

| 8 bytes reserved here for   |

| two DWORD links. One will   |

| be a link for the DOS queue |

| The other for the device    |

| queue                       |

+-----------------------------+

STATUS WORD

15  14  13 12 11 10  9   8   7  6  5  4  3  2  1  0

+---+---+--+--+--+--+---+---+--+--+--+--+--+--+--+--+

| E |               | B | D |                        |

| R | RESERVED      | U | O | ERROR CODE (bit 15 on) |

| R |               | I | N |                        |

+---+---+--+--+--+--+---+---+--+--+--+--+--+--+--+--++

The status word is zero on entry and is always set by the

driver interrupt routine on return.

Bit 8 is the done bit, it means the operation is complete.

For the  moment  the Driver just sets it to one when it

exits, in the future this will be set by  an asynchronous

interrupt  routine  to tell the DOS the operation is

complete.

DEVICE ERRORS

Bit 15 is the error bit, if it is set then the low 8 bits

indicate one of the following errors:

0 Write Protect violation

1 Unknown Unit

2 Not ready

3 Unknown command

4 CRC error

5 Bad  Request Structure length

6 Seek error

7 Unknown media

8 Sector not found

9 Printer out of paper

A Write Fault

B Read Fault

C General Failure

Bit 9 is the busy bit which is set only by status  calls

(see STATUS CALL below).

DEVICE DRIVER FUNCTIONS AND REQUEST STRUCTURES

INIT (Command Code 0)

Character and Block Devices

+------------------------------------+

| 13-BYTE  Request Header            |

+------------------------------------+

| BYTE # of units                    |

+------------------------------------+

| DWORD Break Address                |

---+------------------------------------+---

| DWORD Pointer to BPB array         |

| (not set by Character devices)     |

+------------------------------------+

One  of  the  functions  defined  for  each device  is

INIT.  This routine is called once  when  the  device  is

installed, and  never  again.

Default Character Devices:

No parameters passed, none returned

Installable Character Devices:

If the INIT call is being  made  for  an installable

driver,  the DWORD Pointer to  the  BPB  array at the

time of the call,  offset 18 will be pointing to the

address in memory of the "="  in the "DEVICE ="

parameter in CONFIG.SYS. This  allows  the  CONFIG.SYS

file to pass parameters to the device.

The BREAK ADDRESS parameter must be returned this is a

pointer to the first available byte of memory above the

driver.  This may be used to throw away initialization

code.

Default Block Devices

Return a one byte value giving the number of subunits

which  determines logical device  names.  If the

current maximum logical device letter is F at the time

of  the  install  call, and the init routine returns 4

as the number of units, then they will have logical

names  G,  H,  I  and  J. This mapping  is  determined

by  the  position of the driver in the device list and

the number of units defined by the  device (stored in

the first byte of the device name  field).  Despite the

fact that the number of units  is  stored in the device

header,  you must  still return this number at offset

13D in the INIT call.

A pointer to an array of WORD offsets to BPB's (BIOS

Parameter Blocks) is also returned.  There must be one

entry  for each unit defined by the  device  driver.

If  your Device Driver is defining 2 units,  then the

DWORD  pointer points to the first of two one word

offsets which  in turn  point to BPB's. If both BPB's

are  the  same,  this will allow you just to enter two

offsets to the same  location, saving space. The  BPB

is  used  by  the DOS  to  create  an  internal

structure called the Drive Parameter Block (DPB).

Differences for Installable Drivers

Instalable drivers must return the BREAK ADDRESS free

memory pointer.  The array of BPB pointers, as  well

as the BPB's must be protected - that is below  the

address returned.    In the  case of an installable

device,  the Drive Parameter Block  will  be built

immediately following the  device  driver.   The  BPB's

defined at boot time are scanned to  determine  the

largest sector size which is then used to  determine

the  cache buffer size.  Installable block devices  can

not have  greater sector sizes than that defined by

the default drivers.  For this reason, you may want to

use dummy  BPB's just to reserve space.  In this  case

make sure you use an invalid media byte so when you

later report the correct BPB for the unit, MS-DOS will

build a DPB.

FORMAT OF BPB (Bios Parameter Block) -

+------------------------------------+

| WORD Sector size in Bytes          |

|  Must be a multiple of  32         |

+------------------------------------+

| BYTE Sectors/Allocation unit       |

|  Must be a power of 2              |

+------------------------------------+

| WORD Number of reserved (boot)     |

|  sectors. May be zero              |

+------------------------------------+

| BYTE Number of FATS                |

+------------------------------------+

| WORD Number of directory entries   |

| in the root directory              |

+------------------------------------+

| WORD Total number of sectors       |

| includes reserved sectors          |

+------------------------------------+

| BYTE Media descriptor              |

| must be unique for each BPB        |

+------------------------------------+

| WORD Number of sectors occupied by |

| a single  FAT                      |

+------------------------------------+

MEDIA CHECK  (Command Code 1)

+------------------------------------+

| 13-BYTE  Request Header            |

+------------------------------------+

| BYTE Old Media Descriptor from DPB |

+------------------------------------+

| BYTE returned                      |

+------------------------------------+

The  MEDIA CHECK function is called any time there is a

pending drive access call other than a file read or write.

The purpose is to determine whether the media in the drive

could have been changed.  Note that the previous media ID

byte is passed to the device driver although even if the

media ID byte is the same, a new disk may now be in the

drive in which case, FAT and directory sectors in memory

will be invalid.  The MEDIA CHECK routine will set the

status word and return a byte with the following values:

Returned Byte

-1 Disk has been changed

0 Don't know if Disk has been changed

1 Disk has not been changed

If the driver can return -1 or 1 (by having a door-lock or

some  other interlock mechanism,  the performance of MSDOS

will be DRAMATICALLY  enhanced since MS-DOS will not have to

reread  the  FAT  for each directory access.  This is an

extremely important factor  and  some OEM's without door

locks on their drives have  resorted  to very creative

techniques to  surmount  this  problem  with  considerable

rewards.  One OEM determined that it was impossible  for

the operator to change a disk in less than 2 seconds

therefore anytime Media Check occurs within 2 seconds of a

disk  access,  the driver reports "1",  media not changed.

This made a tremendous improvement in performance.

When a disk access call (other than a file read or write)

occurs,  the  following sequence of events takes place

within MS-DOS.

1. The drive letter is turned into a DPB pointer by

looking through the list of DPB's until the correct

driver unit number is found.

2. The pointer to the Driver is stored in the DPB and

this is used to request a media check on that sub unit.

MS-DOS passes the old media descriptor byte.  The

driver returns:

Media Not Changed...... (1)

Don't know if changed...(0)

Media Changed...........(-1)

Error

a. If media not changed, proceed with the disk access.

b. If media changed, invalidate all buffers associated with

this  unit and request a  new  Bios  Parameter Block via

BUILD BPB call (step 3).

c. If "don't know", then if there any dirty buffers (disk

sectors which have been modified and not written back out to

the disk yet) for this unit, assume that the disk has not

been changed and proceed.  If there are no dirty buffers for

the unit, do a BUILD BPB device call (step 3).

3. If the media has definitely been changed or it might have

been changed and there are no dirty  buffers,  call the

device and request a new BPB.  Once the BPB has been

returned, build a DPB and proceed with the access.

BUILD BPB  (Command Code 2)

+------------------------------------+

| 13-BYTE Request Header             |

+------------------------------------+

| BYTE Media Descriptor from DPB     |

+------------------------------------+

| DWORD Transfer Address             |

| (points to one sectors worth of    |

|  scratch space or first sector     |

|  of FAT depending on the value     |

|  of the NON IBM FORMAT bit)        |

+------------------------------------+

| DWORD Pointer to BPB               |

+------------------------------------+

As described in the MEDIA CHECK call,  the Build BPB call

may  be  made  at any time that there is a pending disk

access  other  than  a  file read or write and the  media

check  call  indicates  that  the disk has been changed or

might have been  changed  (and there are no dirty buffers).

The Device Driver  returns a pointer to a BPB.  This is

different from the INIT call where a pointer to an array of

word offsets to BPB's is returned.

The  data  passed in the request packet depends on the NON

IBM  FORMAT bit in the attribute word of the device driver

header.  If  the NON IBM FORMAT bit  of  the  device  is

set,  then the DWORD  Transfer  Address  points  to  a  one

sector buffer which can be used for any purpose.   If the

NON  IBM  FORMAT bit is  0, then this buffer contains the

first sector of the FAT; in this case the driver must not

alter this buffer  (this mode is  useful  if  all  that  is

desired is to read the FAT ID byte).

If IBM compatible format  is  used  (NON  IBM  FORMAT  BIT =

0), then it must be true that the first sector of the first

FAT is located at the  same  sector  on  all  possible

media.  This is  because  the FAT sector will be read BEFORE

the media is actually determined.

Prior  to MSDOS 2.0,  the first byte of the  File

Allocation  Table  (FAT)  served as an identifier of the

media in the  drive.   Unfortunately, the byte was

constrained to be within the range F8  through FF.  This did

not afford a sufficient range to define all  possible

media.   In  MSDOS  2.0,  the "media  descriptor  byte"

provides the facility to identify different  media.  The

media desriptor byte can be any value between 0  and  ffH.

It need not be the same as the FAT ID byte.  FAT ID bytes

only have significance for IBM format compatible Block

Device Drivers, and  only  the following values have been

defined and  should  be used:

FE  - 1 side  512 bytes per sector 8 sectors per track

FC  - 1 side  512 bytes per sector 9 sectors per track

FF  - 2 side  512 bytes per sector 8 sectors per track

FD  - 2 side  512 bytes per sector 9 sectors per track

For complete details, see Chapter 3 of the Programmer's

reference manual.  The  values of the media descriptor byte

or the FAT ID byte  have no  significance to the MS-DOS.

They are just passed along to the device  driver  to

facilitate media determination in any way  the OEM choses to

implement.

IMPORTANT:

When the BPB call is made, if the media byte returned

in the new BPB is the same as the old media byte, A NEW

DPB IS NOT BUILT.  Therefore EVERY BPB must have a

unique Media Byte!

In order to allow for many different  OEMs  to  read  each

other's disks,  MICROSOFT suggests that the information

relating to the BPB be  kept in the boot sector of the disk.

The format of the boot sector is:

+------------------------------------+

| 3 BYTE near JUMP to boot code      |

+------------------------------------+

| 8 BYTES OEM name and version       |

---+------------------------------------+---

B  | WORD bytes per sector              |

P  +------------------------------------+

B  | BYTE sectors per allocation unit   |

+------------------------------------+

|  | WORD reserved sectors              |

V  +------------------------------------+

| BYTE number of FATs                |

+------------------------------------+

| WORD number of root dir entries    |

+------------------------------------+

| WORD number of sectors in logical  |

^  | image                              |

|  +------------------------------------+

B  | BYTE media descriptor              |

P  +------------------------------------+

B  | WORD number of sectors per FAT     |

---+------------------------------------+---

| WORD sectors per track             |

+------------------------------------+

| WORD number of heads               |

+------------------------------------+

| WORD number of hidden sectors      |

+------------------------------------+

The three  words  at the end are optional, MS-DOS doesn't

care about them (since they are not part of  the  BPB). They

are intended  to  help the BIOS understand the media.

Sectors per track may be redundant (could be figured  out

from  total size of  the  disk).  Number of heads is useful

for supporting different  multi-head  drives  which  have

the  same  storage capacity, but  a  different  number  of

surfaces.   Number of hidden sectors is useful  for

supporting  drive  partitioning schemes.

Determination of BPB's by NON IBM FORMAT drivers

Since  pre-msdos 2.0 diskettes do not have BPB's in  the

boot sector,  the  following procedure is recommended for

determining the media.

1. Read the boot sector of the drive into the 1 sector

scratch space pointed to by the DWORD transfer

address.

2. See if the first byte of the boot sector  is an E9

(the first byte of a 3 byte jump). If so, the BPB is

there.  Return a pointer to it.

3. If the boot sector does not have a BPB table, it

must  be a pre-2.0 IBM format disk which uses a FAT ID

byte.  Read the FAT into the 1 sector scratch area and

read the first byte to determine which of the four

recognized 5 1/4" formats are present.  Return a

pointer to a hard-coded BPB.

READ or WRITE  Including IOCTL (Command Codes 3,4,8,9,12)

+------------------------------------+

| 13-BYTE  Request Header            |

+------------------------------------+

| BYTE Media descriptor  from DPB    |

+------------------------------------+

| DWORD transfer address             |

+------------------------------------+

| WORD byte/sector Count             |

---+------------------------------------+---

| WORD starting sector number        |

|  (ignored on Char Devs)            |

+------------------------------------+

The  Driver  must  perform the Read or Write depending  on

which  command  code is set.  Block devices will read or

write  sectors,  character devices will read or write bytes.

Upon completion  of the  I/O,  the device driver must set

the status word AND  REPORT  THE  NUMBER OF SECTORS OR BYTES

SUCCESSFULLY  TRANSFERRED.  This  should  be done even if an

error prevented the transfer from being completed.  Setting

the error bit alone is not sufficient.  NOTE:  No error

check  is performed  on an IOCTL  I/O call,  driver MUST

correctly set the return sector (byte) count  to  the actual

number  of  bytes  transferred, however.

If  the verify switch is on,  the block device driver will

be  called  with  command code 9 (write with  verify). Your

device  driver will be responsible for verifying the write.

NOTE: THE FOLLOWING APPLIES TO BLOCK DEVICE DRIVERS.

Under certain  circumstances  the  BIOS  may  be  asked to

do a write operation of 64K bytes which seems to  be  a

"wrap around" of  the transfer address in the BIOS I/O

packet.  This arises due to an optimization  added  to  the

write  code  in MS-DOS.  It will only occur during user

WRITEs which are within a sector size of 64K bytes on files

which are  "growing"  past the current  EOF.   IT  IS

ALLOWABLE  FOR  THE BIOS TO IGNORE THE BALANCE OF  THE

WRITE  WHICH  "WRAPS  AROUND"  IF  IT  SO CHOOSES.   For

instance  a  WRITE  of  10000H  bytes worth of sectors with

a transfer address  of  XXX:1  could  ignore  the last two

bytes (remember that a user program can never request an I/O

of more than FFFFH bytes and cannot wrap  around  (even to

0)  in  his  transfer segment, so in this case the last two

bytes can be ignored).

NON DESRUCTIVE READ NO WAIT - (Command Code 5)

+------------------------------------+

| 13-BYTE Request Header             |

+------------------------------------+

| BYTE read from device              |

+------------------------------------+

The device sets the DONE BIT.  The BUSY BIT in the status

word is set as follows:

Busy bit = 0   characters in  buffer

the  next  character  that would be  read  is

returned.  This  character is NOT removed from the

input buffer (hence the term Non  Destructive

Read). In essence  this  call  allows the DOS to

look ahead one input character.

STATUS Calls (Command Code 6, 10)

+------------------------------------+

| 13-BYTE Request Header             |

+------------------------------------+

All driver must do is  set  status  word  accordingly  and

set the busy bit as follows:

For output on  character  devices:

BIT = 1   On return, a  write  request  (if  made) would

wait for completion of a current request.

BIT = 0   There is no current  request  and a write request

(if made) would start immediately.

For input on character devices with a buffer, a  return of 1

means  a  read request (if  made) would go to the physical

device.  If it is  0  on  return,  then there are

characters  in  the device's  buffer  and a read would

return  quickly, it also  indicates  that the user  has

typed  something.

MS-DOS assumes all character devices have an  input  type

ahead  buffer. Devices which  don't have them should always

return busy = 0 so  that the DOS won't hang waiting  for

something to get into a buffer which doesn't exist.

FLUSH Calls (Command Codes 7, 11)

+------------------------------------+

| 13-BYTE Request Header             |

+------------------------------------+

This  call  tells  the  driver  to  flush  (terminate) all

pending requests  that  it  has knowledge of.  Its primary

use is to flush the input queue on character devices.  All

pending requests  that  it  has knowledge of.  Its primary

use is to flush the input queue

SYSINIT AND DOS INITIALIZATION

In addition to coding the default drivers and performing any

hardware initialization, your IO.ASM (BIOS) code must

initialize certain external variables which are declared

public in the SYSINIT module to which the IO.OBJ module is

linked.  The public variables and their functions are

described below.

CURRENT_DOS_LOCATION     WORD

This is the paragraph location  where your  Bootstrap loader

(or optionally your BIOS) has loaded DOS  into memory.

Typically, the DOS will be located immediately after IO.SYS.

FINAL_DOS_LOCATION       WORD

Since SYSINIT immediately relocates itself to high memory,

there will be a hole right after the default drivers.  By

telling SYSINIT where you want it to relocate the DOS, you

can fill up this hole plus overlay any  code that the BIOS

used for BOOT time hardware initialization.  SYSINIT will

move the DOS down to this location conserving space.

(Address in paragraphs)

DEVICE_LIST         DWORD

You must tell SYSINIT where the beginning of the linked list

of Device Drivers Starts.  This information will in turn be

passed to MS-DOS.  This is the location of the CON device

driver  header.

MEMORY_SIZE         WORD

MSDOS should be implemented in systems with contiguous

memory.  This  word is the paragraph number of the highest

location in RAM.  If you do not initialize this value,

SYSINIT will do a memory scan to determine the amount of

memory.  It does this by reading memory, writing an

arbitrary pattern, reading it back for a match and then

returning the original value.  It stops when it gets a

mismatch.  If your system has parity memory, you must

declare this value since writing to non-existent memory will

cause a parity error.  SYSINIT BEGINS THE MEMORY SCAN AT 32K

DEFAULT_DRIVE       BYTE

If your system can boot off of a number of drives, you must

tell SYSINIT which drive you booted off of so it can

properly find CONFIG.SYS and COMMAND.COM.  This variable

should be set accordingly. Drive A = 1, Drive B = 2, etc.

If this value is not set, Drive  A is assumed.

BUFFERS             BYTE

This number is the default number of sector buffers for the

system.  This value may be overridden by the user in the

CONFIG.SYS file.  If this value is not set, it will default

to two.  It should be greater than 1.

FILES               BYTE

This is the default number of open files that the XENIX

system calls can access.  This value may be overridden by

the user in the CONFIG.SYS file.  Its default setting in

SYSINIT is 8.  Values less than 5 are ignored.

The entry point in SYSINIT should be defined as a FAR LABEL

in your BIOS as follows

EXTRN SYSINIT:FAR

After your BIOS code has performed any hardware

initialization and has set the above external variables, it

should do a FAR JUMP to this label.

You must provide a FAR procedure in the IO.ASM code which is

not subject to being overlayed when SYSINIT relocates MS-

DOS.  This procedure is called RE_INIT.  This label should

be declared public as follows:

PUBLIC   RE_INIT:FAR

The procedure should be declared as follows:

XXX  PROC FAR

RE_INIT:

.......

.......

RET

XXX       ENDP

The RE_INIT procedure is called by SYSINIT after MS-DOS is

installed. On entry, DS:0 points to a 100 hex byte program

segment prefix which should be thought of as the header of

the SYSINIT and IO.ASM device driver code taken together.

This is not a normal program in that no memory is allocated

to it therefore if you allocate and load or exec as you

would normally, you will probably overlay this block.

SYSINIT itself is located in the TOP 10K of memory.  DO NOT

DO WRITES THERE.  uThe space above the 100H byte header at

DS:100 is available for temporary use during RE_INIT.

The RE_INIT routine can be used to print headers, read

files, etc.and is handy for BIOS debugging.  If you don't

need to use this procedure simply do a RET.   RE_INIT MUST

NOT CHANGE ANY DOS REGISTERS OR FLAGS.  The ROLE of SYSINIT

in DOS initialization As soon as SYSINIT gains control from

the BIOS initialization code, it determines the size of

memory (performing a scan if requested) and based on the

highest paragraph and the size of SYSINIT, it relocates

itself to high memory.

Running in high memory, it moves DOS from the

CURRENT_DOS_LOCATION to the FINAL_DOS_LOCATION as specifed

by the IO.ASM.  It then does a FAR CALL to MSDOS.  MSDOS has

initialization code which steps through the linked list of

device drivers and performs  the INIT call to each.  The

copyright message is then printed out on the screen.  After

initializing the default devices, building drive parameter

blocks for the default block devices and installing a sector

buffer, MS-DOS does a FAR RET to SYSINIT.

During the MS-DOS initialization, MSDOS examines the Bios

Parameter Blocks returned by the INIT call to the block

devices.  It uses the largest sector size it finds as the

default buffer size for all buffers.  For this reason,  The

initial BPB's returned may not reflect the the currently

installed disks but be used to reserve space for an

installable device with a larger sector size.

SYSINIT then calls RE_INIT in the  BIOS (IO.ASM) to allow

the OEM  to print headers or handle any other initialization

which can now be done via standard MS-DOS system calls.  (Do

NOT attempt to MODIFY MEMORY).

After control returns to SYSINIT from the  BIOS, SYSINIT

attempts to open a file called CONFIG.SYS.  If found, this

file is loaded in memory and all characters turned to

UPPERCASE.  It is then parsed looking for Keywords such as

DEVICE, BUFFERS, SHELL = and so on.  Any new devices are

loaded, added to the linked list and installed via the INIT

call.  Character devices are added to the front of the list,

and new block devices are added to the end.  The presence of

a new CON device, for  instance,  would supplant the

existing default device.  This provides great facility to

completely reconfigure the BIOS at a later time if desired.

SYSINIT performs some "housekeeping" chores and  allocates

all of the memory it needs to protect the buffers, installed

device driver code and BIOS.  It sets the "owner" of the

memory to a special value  to make sure the memory  is never

deallocated.  SYSINIT then closes all file handles* except

for standard error, (handle 2) and then re-opens CON, AUX,

and PRN.  This enables a new CON, AUX or PRN device to

replace the default ones.   SYSINIT allocates memory for the

buffers and then does an OPEN of COMMAND.COM and loads it.

The COMMAND.COM segment and offset are pushed on the stack

and  a final FAR RETURN to COMMAND.COM is made.  MS-DOS is

now up and running.

The first thing that COMMAND.COM will do is allocate memory

for its transient and reload it.  The transient part of

COMMAND which contains the built in commands COPY, DIR,

DATE, TIME, etc resides in unprotected  memory when a user

program is loaded into the transient program area (TPA).  It

will be overlaid by programs needing the space.  When a user

program terminates to the COMMAND resident, the transient is

checked to see if a reload is necessary.

ANATOMY OF A DEVICE CALL

1. The DOS calls the Block Device STRATEGY Entry Point.

2. The Device Driver saves the ES and BX registers (ES:BX

points to the request packet) and does a FAR RETURN

3. The DOS calls the INTERRUPT Entry Point

4. The Device Driver reads the COMMAND CODE (offset 2 in the

request packet) to figure out what kind of request this is.

It turns out to be a WRITE. Through a dispatch table,

control passes to the disk write routine.

5. The Device Driver reads the UNIT CODE (offset 1) to

figure out which Disk Drive is supposed to write.

6. Since the command is a disk write, the Device Driver must

get the TRANSFER ADDRESS (offset 14),the SECTOR COUNT

(offset 18) and the START SECTOR (offset 20)  in the request

packet and read the data.

7. The device driver writes the specified number of sectors

starting at the beginning sector on the  drive defined in

the UNIT CODE, the subunit defined   by this device driver,

transferring data from   the Transfer Address indicated in

the request packet.

8.  After the transfer is complete, the device driver  must

report back to the DOS on the status of the request.   It

does this by setting the DONE bit in the status word

(offset 3 in the request packet).  It reports   in the

SECTOR COUNT area of the request packet the   number of

sectors actually transferred (not necessary   if the number

of sectors same as requested).

9.  The device driver does a FAR Return to the DOS.

NOTE: NO DATA ARE RETURNED IN REGISTERS!

10. HAD AN ERROR OCCURRED, the driver would have set the

DONE bit and the ERROR bit in the status word and  filled in

the error code in the lower half of the  status word.  THE

NUMBER OF SECTORS ACTUALLY TRANSFERRED   MUST BE WRITTEN IN

THE REQUEST HEADER.   It is not sufficient just to set the

ERROR bit of the status word.

The DEVICE DRIVERS SHOULD PRESERVE THE STATE OF THE DOS.

This means that all registers (including flags) should be

preserved.

At the time that the DOS calls the Interrupt Entry point in

the device driver, the DOS has room for about 40 to 50 bytes

on its internal stack.  It is recommended that at least

while initially bringing up the DOS on your machine, you

avoid problems by switching to a local stack for  device

driver use.

3 MS-DOS disks are divided into four logical data areas.

1.  Reserved Area (Boot Sector)  MS-DOS xxxxx the disk

2.  File Allocation Tables in a series of logical sectors

3.  Root Directory logical sectors

4.  Program/Data Area starting with 0 going

up to some number n.

When creating your own disk for xxxx (for non-standard

diskettes or for nonremovable media such as hard disks) the

logical disk which your device driver presents to MS-DOS

must conform to the above standard.  The physical layout may

be completely different.  For example, a number of

manufacturers have implemented a partitioning scheme on the

hard disk to allow it to be shared by multiple operating

systems.  In this case either the device driver or firmware

must take care of mapping the physical structure into an MS-

DOS logical layout.  A few manufacturers have even

implemented the MS-DOS file system logically within another

operating system's file structure.  Once again, in this

case, the responsibility lies with the device driver or

firmware to make it look like an MS-DOS disk format.  .s1

Reserved Area  .s1 The reserved area is a part of the disk

starting at the beginning which MS-DOS will never look at.

This is typically used for boot purposes.  Some machines

which were not designed with MS-DOS in mind may choose to do

something special with track 0.  In this case, the entire

track may be marked as reserved or the device driver may

simply choose to map MS-DOS's logical sector 0 to the

beginning of track 1 instead of track 0.  .s1 Most

implementations will have first the difficulty fitting the

required boot information into a single sector; however,

disk formats utilizing 128 or 250 byte sectors and without

any firmware support may require several sectors.  .s1 How

to Describe Media

Beginning with MS-DOS 3.00, Microsoft is promoting the use

of a media description table in the boot sector. This table

contains both a physical (number of sectors, sides, tracks,

etc.) and a logical (number of FATs, directory entries,

etc.) description of the disk layout.  Refer to the media

description table in the device drivers' chapter of the

programmer's manual for further details.  This media

description table allows MS-DOS to accommodate for diskette

formats which may be defined in the future or for formats

defined by other manufacturers but not generatable by your

format utility.  When your device driver reads the boot

sector for media determination, it will assume that the

table exists if the first byte of the boot sector includes a

near jump (__ __ _H).

What the ROM Must Do With the Boot sector

If the reserved area contains a boot sector (as it should if

the media is bootable) the boot sector has the following

responsibilities: It must read in the first sector of the

directory and verify that the first two files on the disk

are "IO.SYS" and "MSDOS.SYS" in that order.  Please note

that you may choose to have your format utility generate

different boot sectors for each disk format or create one

universal boot sector which reads its own media description

table to determine where to find the first director sector

both logically (the size of the reserved area and size and

number of FATS affect the placement of the directory) and

physically (have the size of the reserved area and FATs

pushed off on to track 1 or the other side.)  If the MS.DOS

files are not found in the directory by the boot code then

it must prompt the operator that he/she has attempted to

boot from a non-bootable diskette and that he/she must

insert a bootable diskette to continue.  It is up to you to

determine how he/she should continue (strike a key or power

off).

The next responsibility of the boot sector is to read in the

"DO.SYS" and "MSDOS.SYS" files which it may assume are

contiguous on the disk.  The boot sector must know or

calculate where those files begin physically on the disk

(right after the last direction sector) and read them in.

You must hand code the (combined) length of these files into

the boot sector.  You must also hand code the location at

which they should be loaded into memory in the boot sector.

The boot sector must report only disk read errors to the

user.  It must then transfer control to the beginning of

IO.SYS.  The boot sector does not need to worry about

COMMAND.COM, as this will be loaded by MSDOS.SYS.  The boot

sector does not need to worry about IO.SYS and MSDOS.SYS

being contiguous as they can only be placed on the disk by

format IS or SYS both of which insure that they will be

contiguous.

In order to be able to SYS MSDOS 2.0 onto old 1.X diskettes

on the IBM PC, SYS was modified to allow the new MSDOS.SYS

to be placed on the diskette in a non-contiguous situation

(IO.SYS would still be contiguous because the new IO.SYS was

still smaller than the combined sizes of the old IO.SYS and

MSDOS.SYS).  This meant that the boot sector was only

responsible for loading in IO.SYS (although it still needed

to check that the MSDOS.SYS was the second file on the

diskette).  IO.SYS then became responsible for loading a

possibly non-contiguous MSDOS.SYS. This meant that logic to

facilitate checking through the FAT entries had to be added

to IO.SYS because this logic normally resides in the not yet

loaded MSDOS.SYS.  This logic was put in the installation

portion of the IO.SYS so as to be overwritten once MSDOS.SYS

was loaded.  This technique should only be used if you need

to be able to SYS in new, bigger MSDOS onto an old disk.  It

provides no advantage if you do not have old disks to

support in the field.  If you are starting with 2.0, ignore

this technique.

Next on the disk comes the File Allocation Table.  Because

this is the most important data structure on the disk the

copies are usually kept.  (It makes no sense to use 2 FATs

for a virtual RAM disk because in this case the media cannot

be damaged.)  The FAT is a linked list of pointers, one for

every allocation unit on the disk plus 2 extra.

FAT size = (3*number at sectors on disk)

+6-(3*number reserved sectors)

-( 3*directory sectors)

sectors)                                     2*(Sector size-

3)#Allocation

The format program will take care of creating a proper

FAT.  (What happens if you tell it extra sectors?)  The FAT

has a l 1/2 byte entry for each allocation unit in the

program area of the disk.  It also contains 2 extra entries

at the beginning.  The first entry is an FFxA. This is an

end-of-file marker and is pointed to by unused FAT entries

(000*). The first byte may be used for media determination.

Trouble Shooting

Problem:

When I type "r" for Retry when a disk error occurs, the

system crashes.

Typical Cause:

When an error occurs during I/O, your device driver

must report thenumber of sectors or characters

correctly transferred.  If you just set the error bit,

the DOS will assume that it was able to read all of the

sectors since the sector count will be the same as set

by the DOS.

Problem:

I am booting up the DOS.  I see the MSDOS copyright

message on the screen, but then I get the message "bad

or missing command interpreter".

Typical Cause:

Sysinit is trying to OPEN COMMAND.COM preparatory to

loading it.  Some kind of disk error has occurred.

Either COMMAND is not on the disk, or perhaps you

returned a bad BPB and the DOS can't find it.

Problem:

I am booting up the DOS.  I get the MSDOS copyright

message on the screen, but then I get the message "Bad

or missing /DEV/CON".

Typical Cause:

Sysinit has closed all file handles and is now

reopening them. The first file it opens is /DEV/CON

which is your console device. The system file table is

examined for this device - it should  be present.  If

it is not found, this indicates that some of yourBIOS

code has trashed the system file table.  This may be

caused because you have BLOWN the MSDOS STACK causing

the DOS to write over the file table.

Problem:

The DOS is trying to read some sectors starting at an

extremely large or nonsensical sector number.

Typical Cause:

The BPB that you returned for the drive is BAD.  Check

the INIT and Build BPB device driver routines.

Problem:

After I jump to SYSINIT, control never comes back.

Typical Cause:

1.   You may not have loaded DOS correctly with your boot

strap loader.  SYSINIT must know exactly where DOS is.

DOS must be on a paragraph boundary.

2.   If you are requesting a memory scan - don't. specify

some value for the highest contiguous paragraph in

memory.

3.   Put a break point or some debugging code in RE_INIT to

see if you are getting this far.  RE_INIT gets

called immediately after DOS relocation and

initialization.

4.   SYSINIT is in the top of memory (as stated) at this

time. Do you have any holes in memory?  If so,

understate memory so SYSINIT has a contiguous piece to

work with.

Problem:

Everything was working great, but when I tried to

install a new device, the system won't boot.

Typical Cause:

You are failing to set the BREAK ADDRESS during the

Device INIT routine.  This points to the first byte of

free memory after the device code and data.

Problem:

Everything is working OK until I define a new drive in

my BIOS.  Then the system won't come up.

Typical Cause:

Look at the INIT call to the block device driver.  This

is probably not being handled correctly.  You are

supposed to return a DWORD pointer to an array of WORD

POINTERS to  BPBs.  There should be one array element

for each subunit that the device driver is defining.

The number of subunits is also returned by the INIT

call.

Problem:

My device driver defines two drives, but when I try to

use drive B:, I get an "Invalid Drive Specification"

message.

Typical Cause:

You are not handling the INIT call to the device driver

properly.

Glossary of MSDOS terminology

ASCIZ

Any Null (Zero) Terminated ASCII string

COOKED and RAW

Character devices have two modes for I/O, "cooked" and

"raw".  Cooked mode is the default, and I/O in cooked mode

is character at a time with control C checking depending on

whether BREAK is ON or OFF and depending on the function

request (1-12 or XENIX reads and writes). RAW mode, on the

other hand, really refers to absolutely RAW I/O.   If you

put a device in raw mode via the 44H function request

defined in the programmers reference manual, there will be

no control C checking, echoing or editing of characters.

For example, if the requesting program puts a device in RAW

mode and requests a write of 50,000 characters to the AUX

device, The Device Driver will  get a request from the DOS

to write 50,000 characters from the buffer locted at the

DWORD transfer address.  The driver will not return until

the request is complete.  Obviously, this I/O will be fast.

In  cooked mode, MSDOS would perform a control C check at

the console after each single character write.  This is an

overhead of 50,000 control C checks.

File Handle

In MSDOS 2.0, there is a system file table set up at boot

time.  By the time command.com gets control, all of the

character devices have been entered into the system file

table.  The first 4 devices are in a specific order.  The

0th entry (file handle 0) is Console Input which is open for

READ ONLY.  File handle 1 is Console Output - WRITE ONLY.

Handle 2 is Standard Error output. Handle 3 is the AUX

device, and  Handle 4 is the PRN device.  As new entries are

made to the system file table, they are assigned the next

numbers.  When COMMAND.COM EXEC's a program, the "CHILD"

program "INHERITS" all of the handles that COMMAND.COM has

open.  IF you type  on the command line: PROG < INFYLE  >

OUTFYLE  This means that you want PROG to read its input

from INFYLE instead of the CONsole and write its output to

OUTFYLE instead of to console out.  COMMAND.COM will close

handle 0 and open INFYLE it will DUP handle  Handle 0.  It

will then close handle 1, standard out and open OUTFYLE

which will now be assigned standard out. COMMAND will exec

PROG which will inherit this environment.  When PROG reads

from standard in, or writes to standard out, it will be

reading from and writing to INFYLE and OUTFYLE respectively.

The 100H byte Program Segment Prefix contains a set of 20  1

byte entries which is an index into the system file table

entries which it has open.  If the first entry in the PSP

has a value of 8, this means that the Program is using entry

8 in the system file table as standard input.  This could be

either a device or a file.  If a child is EXEC'd, it

inherits the files of its parent. The converse is not true.

When a  child program which opens AUX as standard IN and PRN

as standard out returns, the Parent does not inherit the

files of its child.

The ENVIRONMENT

MSDOS 2.0 introduces another feature of XENIX, the

Environment.   The environment is an UP to 32K data area

which consists of  ASCIZ (null terminated) strings of the

form:  AAAA=BBBBBBB.  The end of the  environment is marked

by two consecutive nulls.  The word at 2CH in the program

segment prefix points to the segment containing the start of

the program's environment.  New variables can be added to

the environment by use of the MSDOS SET command.   Since

Nulls have significance, the environment can not be used for

storing Binary data.  When a parent process (such as

command.com) exec's a child process, the child is given a

COPY of the parent's environment.    Parameters such as

PROMPT= which specifies the style of prompt to be used by

COMMAND are stored in the environment.

The ARENA

MSDOS employs a software memory management scheme.  Blocks

in memory are either OWNED by processes or are free.  An

owned block has an ID number which is the initial paragraph

of the Program Segment Prefix of a process in memory (your

program). Each block has a size as well.  When your process

is in memory, there is a 16 byte ARENA BLOCK immediately

below it which identifies it and specifies how much memory

is allocated.

ALLOC's and DEALLOC's as described in the  MSDOS

programmer's reference manual manipulate these blocks.  The

arena blocks form a linked list throughout memory.

The EXE file has a 512 byte header which contains initial

segment offsets and a table of relocation items.  There are,

in addition, two fields which specify how much memory the

file requires.  Currently,  the "maxalloc" field is  set to

FFFF by the linker.  COM files are pure  binary and have no

information stored anywhere to tell the MSDOS loader how

much memory they will require.  Both COM and EXE files, for

different reasons will have all of memory allocated to them.

Any ALLOC call which is not preceeded by a MODIFY memory

call (to free up some memory) will fail.  Simply modify the

amount of memory allocated to your process and then allocs

and execs will succeed.

Media Byte/FAT ID byte

The FAT ID byte is the first byte of the file allocation

table on  the DISK.  Before MSDOS 2.0, it was used to

identify what media was in the drive.  The FAT ID byte was

constrained to be between F8 and FF hex.  This did not

provide a sufficient range to cover all possible media.

MSDOS 2.0 allows OEM's to use a less restrictive technique

to determine the type of media.  It is suggested that the

OEM place a table in the boot sector of the disk.  The table

contains OEM information plus the Bios Parameter Block.

This block contains a media identification byte which can

have any value between 0 and FF Hex.   Neither the media

byte nor the FAT ID byte  have any particular significance

to the DOS except that the FAT ID byte has to be within the

range F8 to FF.

The media byte may or may not have the same value as the FAT

ID byte.  The media byte is passed to the DOS as part of the

Bios Parameter Block and returned during the Media Check and

Build BPB calls. The DOS compares the media byte in the BPB

retured and if it is the same as before, does not build a

Drive Parameter Block.  The significance of the media byte

vs FAT ID byte depends on whether you have decided to make

your Block Device Driver IBM Format Compatible or non IBM

Format  Compatible.  You must indicate non IBM compatility

by  setting bit 13 in the attribute word in the header of

the device driver.  With non-ibm format bit set, you are

afforded considerable flexibility in supporting various

media including IBM Format Media.  If you are  IBM

compatible,  use hard coded FATID bytes to determine the

media in the drive.   The correspondence between FAT ID

Bytes and various media is defined in the Programmer's

Reference Manual.   If you are non-IBM FORMAT compatible,

when your block device gets called by the DOS to do a BUILD

BPB, you are given  a pointer to a one-sector scratch

buffer.  What you should do is to read the boot sector of

the disk into that buffer and check the first byte.  If it

is an E9 Hex, then it is the first byte of a 3 byte jump and

there is a Bios ParameterBlock table in the boot sector.

Read the Media Byte of the BPB table and determine if it is

the same as the old one that was returned by the DOS.  If it

is then all you have to do is to set the done bit and

return.  The DOS will look at the returned media byte and

since it has not been changed, it will not build a new Drive

Parameter Block.  If the media byte is different, you must

change the media byte in the request packet and give the DOS

a  DWORD pointer to the BPB in the scratch sector.  If when

you looked at the boot sector, you did not see an E9 as the

first byte, there is no BPB table in the BOOT Sector.  You

will then have to rely upon reading the FATID byte.  The FAT

ID byte is the first byte of the FAT which is the second

sector of the disk if it is IBM format compatible.  You then

read the second sector into the scratch pad buffer and

based upon the first byte in that sector, you determine

which of the  currently defined BPB's should be used to

read the disk.  Consult the Programmer's Reference Manual

for a list of the Currently Defined FATID bytes.  These are

the only ones that should be used.  Any other media should

be defined in the boot sector and should use a FAT ID byte

of FFH

###################