VAX Virtual Disk

This system will be made up of the following :

On the VAX running under UNIX there will be 3 programs -

- Format : This generates the boot sector (with BPB etc.), FAT and

Directory sector files. The other data sectors will be

generated on demand by the server program.

- Server : This is a disk server program which communicates with

the block device driver on the PC. The packet format

is as below :

------------------------------------------------------------------

SOH (^A) | Sequence # (2 ASCII digits) | Command#(1 ASCII Digit) |

------------------------------------------------------------------

------------------------------------------------------------------

DATA (Encoded as below) | EOP (^W) | 2 ASCII digits for checksum |

------------------------------------------------------------------

The data encoding is to convert control and binary characters

to harmless printing characters which can be transmitted without problem over the RS-232 channel and through Bridge. It is accomplished with the following scheme :

.	Use 7dH and 7eH as escape characters.

.	20H to 7cH transmitted as is.

.	7fH to dbH translated to 7dH, 20H to 7cH.

.	dbH to 1aH translated to 7eH, 20H to 7cH.

.	7dH 		 translated to 7dH,7dH.

.	7eH 		 translated to 7eH,7eH.

There are 3 command codes defined at present. Command code 0

for Reply. Command code 1 for send sector. Command code 2 for recieve sector. Command codes 1 and 2 have a 3 ASCII digit sub-field for logical sector number.

- Dos2Ux : This makes up the last utility on the VAX to support

the VAX virtual disk. This reads the directory and FAT

sector files on the VAX virtual disk and  generates a

Unix file.

On the PC there is a block device driver specified in the config.sys for installation at boot. The following device driver command perform the indicated function :

0 - Initialization : Installs the RS-232 interrupt driver and puts

the console in direct communication with the

port for login and server invocation as

needed. This may later be enhanced where such

interaction is specified in a script file and

is performed automatically at installation.

1 - Media check    : Local response to the DOS with media not

changed response.

2 - Build BPB			   : Communicates with the VAX server and fetches

the boot sector (command code 1) and extracts

the BPB.

3 - IOCTL input    : this is an unsupported character mode command

and returns with error.

4 - Input			    : Request broken up into a sequence of single

sector transfer request from the  VAX.

5 - Non destructive : Unsupported character mode command. Return

Input (no wait)   with error.

6 - Input status	   :          - do -

7 - Input flush     :          - do -

8 - Output          : Request broken up into a sequence of single

sector transfers to VAX.

9 - Output with     : Executed as Output (command 9)

verify

10 - Output status  : Unsupported character mode command. Error

return.

11 - Output flush   :           - do -

12 - IOCTL output   :           - do -

13 - Device Open    : Local dummy return.

14 - Device close   : Local dummy return.

15 - Removable media: Return with busy status (bit 9) indicating

non-removable media.